Safe Hearing

Individuals who have difficulty hearing due to hearing disabilities often face safety challenges in urban environments, where sound cues (like car horns, sirens, construction noises, or animal sounds) signal danger. One of our project ideas aims to bridge the sensory gap by creating an audio classification system that detects and identifies hazardous street sounds and reports a label, confidence, and a risk tier in real time. The system could eventually be integrated into a mobile app that visually or haptically alerts the user of any potential risks.

Our expected outcome is a signal classifier capable of identifying and categorizing hazardous sounds versus non-hazardous sounds, assigned a class label(e.g.siren/horn/alarm/engine), a calibrated confidence, and a risk tier(low/med/high) to balance missed alarms and false alerts.

We plan on using several key concepts and tools from class and outside to achieve our goal:

- Filter and Sampling: Bandpass filters to isolate human-audible frequencies and reduce noise

- Discrete Time Fourier Transform (DTFT): For spectral feature extraction

We will extract a small, concrete feature set: MFCCs, log-Mel spectrogram bins, short-time energy, and spectral peaks/harmonicity to capture siren/horn patterns. For models, we will start with a lightweight SVM on aggregated features, and a tiny CNN on log-Mel as an optional upgrade if time permits. A brief temporal smoothing will stabilize decisions before triggering alerts.

We would plan to use publicly available (but reputable) datasets. Both the current datasets we have found are from a Kaggle repository. We can spend more time searching for more data if these prove to be unusable. We will strictly follow official folds to prevent leakage and address class imbalance via class weights or resampling.

- UrbanSound8k : A large set of sounds that are naturally found in urban settings

- ESC-50 : a public collection of environmental sound classification

We plan on using mainly audio tools for this project idea. We intend to use an audio recorder. Potential audio recorders could include iPhone, Airpods, stand microphone, or other general use headsets. We also plan on using a decibel monitor in order to detect how loud the audio from the microphone is. These tools will work hand in hand with data collection for frequencies and amplitudes, characterizing these signals for the purpose of detecting potential hazards and notifying the individual.